TLBC D95 CANAL

BLOCKAGE DETECTION REPORT

AI-Powered Drone Survey Analysis

Raichur District, Karnataka

| 157 Canal Reaches Analyzed | 87.6% Model Accuracy (IoU) |
| --- | --- |
| 64 GB Drone Survey Data | 4 Stage AI Pipeline |

| YOLOv8 Segmentation | Mask R-CNN FIC | DSM/DTM Silt Depth | GeoTIFF Output |
| --- | --- | --- | --- |

Researcher: Prabhakar Yadav   |   prabhakaryadav0505@gmail.com   |   May 30, 2026

What This Report Contains

| Section | Contents |
| --- | --- |
| Pages 1-2: Overview | Project summary, study area, key findings at a glance |
| Pages 3-4: Data & Pipeline | 64 GB dataset description and AI pipeline flowchart |
| Pages 5-6: Methods | Canal detection, blockage classification, FIC damage, silt depth |
| Pages 7-8: Results & Notebooks | Model performance, analysis results, notebook usage guide |
| Pages 9-10: Outputs & Next Steps | File guide, QGIS steps, action items, conclusions |

Why This Survey Matters

The TLBC D95 canal supplies irrigation water to thousands of acres of farmland in Raichur District. Blockages, structural damage, and silt buildup directly reduce water delivery efficiency and crop yields. This AI-powered drone survey provides the first complete, GPS-accurate health assessment of the entire canal network -- enabling field teams and engineers to prioritise targeted repairs that maximise water delivery at minimal cost.

| Survey Location Raichur, Karnataka India | Canal Network TLBC D95 157 Reaches Analyzed | AI Pipeline 4 Stage Deep Learning May 2026 |
| --- | --- | --- |

Project Overview

The TLBC D95 canal system serves thousands of farmers in Raichur district. This project uses deep learning and drone imagery to automate inspection of the entire canal network — replacing weeks of manual surveys with an accurate, GPS-referenced AI analysis completed in days.

Three Core Problems Detected

| VEGETATION BLOCKAGE Dense plants blocking water flow. Found in 35 reaches (22%). 12 sections fully blocked. | STRUCTURAL DAMAGE Cracks, erosion and concrete degradation. 8 critical locations flagged for repair. | SILT ACCUMULATION Sediment buildup measured via DSM-DTM. High silt zones (>1m) reduce canal capacity. |
| --- | --- | --- |

Study Area & Survey Details

| Parameter | Value |
| --- | --- |
| Location | Raichur District, Karnataka |
| Drone Platform | DJI Phantom 4 RTK |
| Ground Resolution | ~3 cm / pixel (very high detail) |
| Total Data | 64 GB (orthomosaics + DSM + DTM) |
| Coverage | Complete D95 canal network |
| Coordinate System | WGS84 UTM Zone 43N (EPSG:32643) |

Key Findings at a Glance

| 122 Clear Reaches | 23 Partial Blockage | 12 Fully Blocked | 8 Structural Damage | 1.66 GB Output Files |
| --- | --- | --- | --- | --- |

Data Description

64 GB of high-resolution drone survey data collected over the complete TLBC D95 canal system. Three data types — orthomosaics, DSM, and DTM — feed the four AI pipeline stages.

Dataset Breakdown (64 GB Total)

| Data Type | Files | Size | Purpose |
| --- | --- | --- | --- |
| Orthomosaics | 5 GeoTIFF files | 59 GB | High-res drone stitched imagery |
| DSM | TLBC_DSM_DTM/ | 2.4 GB | Surface elevation incl. silt and water |
| DTM | TLBC_DSM_DTM/ | 2.4 GB | Bare earth elevation for silt calc |
| Annotations | canal_v5 + annotation_v2 | 2.78 MB | Model training labels |

Orthomosaic Files (Kept Locally — NOT for upload due to size)

| File Name | Size | Area Covered |
| --- | --- | --- |
| TLBC_D95_ATTANUR_1_ortho.tif | 4.2 GB | Attanur Section 1 |
| TLBC_D95_ATTANUR_2_ortho.tif | 22.5 GB | Attanur Section 2 |
| TLBC_D95_ATTANUR_3_ortho.tif | 17.7 GB | Attanur Section 3 |
| TLBC_D95_ATTANUR_4_ortho.tif | 2.9 GB | Attanur Section 4 |
| TLBC_D95_SHAKAPUR_ortho.tif | 10.3 GB | Shakapur Section |

Training Annotations

| Dataset | Size | Used For |
| --- | --- | --- |
| canal_v5 | 680 KB | YOLOv8 canal detection training |
| annotation_v2.json | 2.1 MB | Mask R-CNN structural damage labels |

Data Preprocessing Pipeline

| Step | Description |
| --- | --- |
| 1. Tile Extraction | Orthomosaics split into 1024x1024 px tiles -- notebook 04_patch_tiling |
| 2. Canal Cropping | Canal regions extracted from full orthomosaic using boundary outlines -- 03_canal_extraction |
| 3. Annotation Load | canal_v5 YOLO polygon format + annotation_v2.json COCO format loaded for each model |
| 4. Train/Val Split | 80% training / 20% validation split applied -- prevents overfitting |

Spatial Reference System

| Parameter | Value |
| --- | --- |
| Projection | Universal Transverse Mercator (UTM) Zone 43N |
| Datum | WGS84 -- World Geodetic System 1984 |
| EPSG Code | EPSG:32643 -- all output GeoTIFFs use this system |
| Ground Resolution | 3 cm per pixel (very high detail drone survey) |

AI Pipeline Architecture

Four sequential stages transform raw 64 GB drone imagery into actionable canal health maps. Each stage's output feeds directly into the next.

End-to-End Processing Flow

| INPUT | PROCESSING STAGE | OUTPUT |
| --- | --- | --- |
| 59 GB Orthomosaics 1024x1024 tiles | STAGE 1: CANAL DETECTION YOLOv8 Segmentation v5 Model: canal_detection_v5.pt IoU Score: 0.876 (87.6%) | *_canal_mask.tif Binary canal boundary masks |
| Canal mask + RGB tile | STAGE 2: BLOCKAGE CLASSIFICATION Vegetation Analysis Algorithm Green Coverage + NDVI + Texture Classes: Clear / Partial / Blocked | *_blockage.tif Values: 1=clear 2=partial 3=blocked |
| Canal regions + annotation_v2.json | STAGE 3: FIC DAMAGE DETECTION Mask R-CNN + ResNet50 Model: fic_detector.pth Train: 0.23 Val: 0.18 | canal_data.json structural_damage: true/false |
| DSM (2.4 GB) + DTM (2.4 GB) | STAGE 4: SILT DEPTH MEASUREMENT Elevation Differencing Formula: Silt = DSM - DTM Classes: Low / Medium / High | *_silt.tif + *_silt_depth.tif Depth in metres |

Stages run sequentially. Stage 1 output feeds Stage 2, and so on. All outputs are georeferenced GeoTIFFs in WGS84 UTM Zone 43N ready for QGIS.

Processing Summary

| 64 GB Raw Input Data | 7 Notebooks Run | 157 Reaches Analyzed | 1.66 GB Final Outputs |
| --- | --- | --- | --- |

Methodology: Canal Detection and Blockage

STAGE 1: Canal Boundary Detection

YOLOv8 Segmentation v5 was trained on 680 KB of annotated canal polygons to identify precise canal boundaries in 1024x1024 px image tiles. The model outputs pixel-level masks that are fully georeferenced and align with real GPS coordinates.

| Field | Value |
| --- | --- |
| Model Architecture | YOLOv8 Segmentation Version 5 |
| Training Data | canal_v5 -- 680 KB of polygon annotations |
| Input Tile Size | 1024 x 1024 pixels |
| IoU Score | 0.876 (87.6% overlap with ground truth -- Excellent) |
| Output File | *_canal_mask.tif -- binary canal boundary raster |
| What IoU means | Predictions match expert annotations 87.6% -- production ready |

STAGE 2: Blockage Classification

A rule-based algorithm analyses vegetation within each detected canal segment using three spectral metrics. Results are explicit and explainable -- no black box.

| Metric | What It Measures | Thresholds |
| --- | --- | --- |
| Green Coverage % | Vegetation vs open water surface | Less than 20 = Clear ¦ 20-50 = Partial ¦ Over 50 = Blocked |
| NDVI Index | Vegetation density and health | Below 0.3 = Clear ¦ 0.3-0.5 = Partial ¦ Above 0.5 = Blocked |
| Texture Analysis | Rough plants vs smooth water | Low texture = clear ¦ High texture = vegetation |

Blockage Classification Legend

| Class | Color in QGIS | Vegetation | Required Action |
| --- | --- | --- | --- |
| 1 -- Clear | Green | Less than 20% | No action needed |
| 2 -- Partial | Orange | 20 to 50% | Clear within 2-3 months |
| 3 -- Blocked | Red | Over 50% | URGENT -- clear immediately |

Methodology: Damage Detection and Silt Measurement

STAGE 3: Structural Damage Detection (FIC)

Mask R-CNN with ResNet50 backbone detects cracks, erosion, and concrete degradation. Trained on 2.1 MB of expert-labeled FIC annotations. Detection threshold set at 0.2 for higher sensitivity -- better to flag for review than miss real structural damage.

| Field | Value |
| --- | --- |
| Architecture | Mask R-CNN + ResNet50 backbone |
| Training Data | annotation_v2.json -- 2.1 MB FIC damage labels |
| Detection Threshold | 0.2 (lowered from default 0.4 for higher sensitivity) |
| Morph. Filter | 3x3 kernel, min area 20 pixels |
| Training Loss | 0.23 -- model converged well |
| Validation Loss | 0.18 -- lower than train loss, no overfitting |
| Damage Locations | 8 reaches flagged with structural_damage = true in canal_data.json |

STAGE 4: Silt Depth Measurement

Silt is measured by subtracting bare-earth elevation (DTM) from surface elevation (DSM). Where DSM > DTM inside canal boundaries, the difference equals silt depth in metres. Only canal pixels (from Stage 1 mask) are included to avoid false positives.

FORMULA: Silt Depth (metres) = DSM - DTM

| Class | Depth Range | QGIS Colour | Action Required |
| --- | --- | --- | --- |
| Low (1) | Less than 0.5 m | Light Yellow | Monitor only |
| Medium (2) | 0.5 m to 1.0 m | Orange | Schedule dredging within 3 months |
| High (3) | Greater than 1.0 m | Dark Red | Immediate dredging required |

Results and Model Performance

Model Accuracy Summary

| Model | Metric | Score | Assessment |
| --- | --- | --- | --- |
| Canal Detection v5 | IoU Score | 0.876 | Excellent -- production ready |
| FIC Detector | Train Loss | 0.23 | Converged well |
| FIC Detector | Val Loss | 0.18 | No overfitting -- generalises well |

Blockage Analysis -- 157 Canal Reaches

| Status | Count | Percentage | Required Action |
| --- | --- | --- | --- |
| TOTAL | 157 | 100% | -- |
| Clear | 122 | 77.7% | No action needed |
| Partial Blockage | 23 | 14.6% | Clear within 2-3 months |
| Fully Blocked | 12 | 7.6% | URGENT -- clear immediately |
| Structural Damage | 8 | 5.1% | Engineering inspection required |

Structural Damage: 8 Locations Detected

Canal reaches flagged by FIC detector show cracks, erosion, or concrete deterioration. Load canal_data.json and filter for structural_damage = true to get GPS coordinates for each. These locations need engineering assessment before further deterioration causes canal failure.

PROJECT IMPACT: This automated system replaces weeks of manual surveys. Field teams now have GPS-accurate locations for all 35 problem reaches. Engineers have precise coordinates for 8 repair sites. Analysis completed in days, not weeks.

Pipeline Notebooks

7 Jupyter notebooks implement the full pipeline. Must be run in the exact order shown -- each notebook's output feeds the next stage.

| Notebook | Purpose | Output |
| --- | --- | --- |
| 03_canal_extraction | Crops canal regions from large orthomosaics | Focused canal tiles |
| 04_dataset | Builds training dataset from canal_v5 annotations | Train/val image + mask pairs |
| 04_patch_tiling | Splits images into 1024x1024 px tiles for models | Thousands of image tiles |
| 05_binary_training | Trains YOLOv8 canal detector | canal_detection_v5.pt IoU=0.876 |
| 06_fic_training | Trains Mask R-CNN damage detection model | fic_detector.pth ValLoss=0.18 |
| 07_blockage_detection | Runs full inference and classification | 1.6 GB blockage GeoTIFFs |
| 08_silt_detection | DSM minus DTM silt depth measurement | 57 MB silt GeoTIFFs |

CRITICAL -- Notebook 07 Cell Execution Order Cell 8 (visualization) MUST run AFTER: Cell 1 -> Cell 2b -> Cell 3 -> Cell 4 Reason: Cell 3 creates 'results' dict ¦ Cell 4 adds 'status' + 'structural_damage' Running Cell 8 alone will cause NameError -- the variables do not exist yet!

Notebook 07 -- Cell Dependencies

| Cell | What It Creates |
| --- | --- |
| Cell 1 | Loads canal_detection_v5.pt model into memory |
| Cell 2b | Builds canal_data dictionary with reach geometries |
| Cell 3 | Runs YOLOv8 inference -- creates 'results' dictionary |
| Cell 4 | Runs FIC detector -- adds 'status' and 'structural_damage' fields |

Output Files Guide

Blockage Outputs  --  outputs/07_blockage/  [1.6 GB]

| File | Contains | How to Use |
| --- | --- | --- |
| *_blockage.tif | Pixel values: 0=background 1=clear 2=partial 3=blocked | QGIS: Symbology > Paletted. Set 1=green, 2=orange, 3=red. Overlay on ortho at 60% opacity. |
| *_canal_mask.tif | Binary: 1=canal detected 0=not canal | Verify detection accuracy. Export as shapefile for precise boundary mapping. |
| canal_data.json | reach_id, status, structural_damage flag, GPS polygon coords for all 157 reaches | Filter status='blocked' or structural_damage=true to generate field team task list. |

Silt Outputs  --  outputs/08_silt/  [57 MB]

| File | Contains | How to Use |
| --- | --- | --- |
| *_silt.tif | Classified: 1=low less than 0.5m 2=medium 0.5-1m 3=high over 1m | QGIS: Apply yellow-orange-red gradient. Red zones need immediate dredging. |
| *_silt_depth.tif | Continuous depth values in metres (raw) | Calculate dredging volume. Track accumulation over repeat surveys. Cost estimation. |

Upload Status

| Category | Contents | Size |
| --- | --- | --- |
| UPLOAD TO DRIVE (share with team) | outputs/ + all notebooks + annotation/ + annotation_v2/ | 4.5 GB |
| KEEP LOCALLY (_keep_locally/) | Orthomosaics + DSM/DTM + manual_labels + .claude/ | 59.5 GB |

QGIS Colour Rendering Reference

| File Type | Pixel Value | QGIS Colour / Action |
| --- | --- | --- |
| *_blockage.tif | 1 = Clear Canal | Green #00AA00 -- no action needed |
| *_blockage.tif | 2 = Partial Blockage | Orange #FF8C00 -- clear within 3 months |
| *_blockage.tif | 3 = Fully Blocked | Red #DC143C -- URGENT immediate clearing |
| *_silt.tif | 1 = Low (less 0.5m) | Light Yellow #FFFACD -- monitor only |
| *_silt.tif | 2 = Medium (0.5 to 1m) | Orange #FFD700 -- schedule dredging |
| *_silt.tif | 3 = High (over 1m) | Dark Red #8B0000 -- immediate dredging |

File Naming Pattern

| Pattern | Example and Meaning |
| --- | --- |
| TLBC_D95_{AREA}_{N}_ortho.tif | TLBC_D95_ATTANUR_1_ortho.tif -- raw orthomosaic input |
| {AREA}_{N}_canal_mask.tif | ATTANUR_1_canal_mask.tif -- binary canal boundary raster |
| {AREA}_{N}_blockage.tif | ATTANUR_1_blockage.tif -- 3-class blockage raster |
| {AREA}_{N}_silt.tif | ATTANUR_1_silt.tif -- 3-class silt depth raster |

Conclusion and Next Steps

This project deployed a four-stage AI pipeline to inspect the entire TLBC D95 canal network. Processing 64 GB of drone data, the system identified all blockage zones, structural damage sites, and silt accumulation areas with GPS accuracy -- results are immediately deployable by field teams and engineers without further analysis.

Deliverables Summary

Deliverable

Canal detection model: 87.6% IoU -- production-ready for new drone surveys

Damage detection model: Val loss 0.18 -- generalises well to unseen canal sections

1.6 GB blockage GeoTIFFs: pixel-level classification of all 157 reaches

57 MB silt GeoTIFFs: depth classification + continuous values for dredging estimates

Priority Action Items

| Priority | Action | Deadline |
| --- | --- | --- |
| HIGH | Deploy field teams to 12 blocked reaches -- clear vegetation | Immediate |
| HIGH | Engineering assessment of 8 structural damage locations | Within 2 weeks |
| MEDIUM | Dredge high silt zones (over 1m depth) to restore capacity | Within 3 months |

QGIS Quick-Start

| Step | Action |
| --- | --- |
| 1 | Layer > Add Raster > load orthomosaic as base reference layer |
| 2 | Layer > Add Raster > load *_blockage.tif as overlay |
| 3 | Symbology: Paletted/Unique Values -- set 1=green, 2=orange, 3=red, 0=transparent |
| 4 | Set layer transparency to 60% -- see ortho and classification together |

Prabhakar Yadav   |   prabhakaryadav0505@gmail.com   |   May 30, 2026

TLBC D95 Canal Blockage Detection -- AI-Powered Infrastructure Monitoring, Raichur District